CALCULATIONS

Calculation of the modulation index (μ)

Time domain representation

When the Time domain representation is used,

μ= XAM/MAX-XAM/MINXAM/MAX+XAM/MIN

a). Sinusoidal Signal

- When μ= 0.25,

From figure 01,

XAM/MAX= 750 mV

XAM/MIN =450 mV

μ= 750-450750+450

μ=0.25

- When μ= 0.5,

From figure 02,

XAM/MAX= 900 mV

XAM/MIN = 300 mV

μ= 900-300900+300

μ=0.50

- When μ= 1.0,

From figure 03,

XAM/MAX= 1175 mV

XAM/MIN =25 mV

μ= 1175-251175+25

μ=0.96

b) Square Wave Signal

- When μ= 0.25,

From figure 07,

XAM/MAX= 700 mV

XAM/MIN = 425 mV

μ= 700-425700+425

μ=0.24

- When μ= 0.5,

From figure 09,

XAM/MAX= 900 mV

XAM/MIN = 300 mV

μ= 900-300900+300

μ=0.50

- When μ= 1.0,

From figure 08,

XAM/MAX= 1175 mV

XAM/MIN =25 mV

μ= 1175-251175+25

μ=0.96

c) Triangular Pulse Signal

- When μ= 0.25,

From figure 15,

XAM/MAX= 750 mV

XAM/MIN = 450 mV

μ= 750-450750+450

μ=0.25

- When μ= 0.5,

From figure 17,

XAM/MAX= 900 mV

XAM/MIN =300 mV

μ= 900-300900+300

μ=0.50

- When μ= 1.0,

From figure 16,

XAM/MAX= 1175 mV

XAM/MIN =25 mV

μ= 1175-251175+25

μ=0.96

Frequency domain representation

When the frequency domain representation is used,

μ= 2×sideband RMS value Carrier RMS voltage

PC=Carrier Power

PS=Sideband Power

AC,RMS=RMS voltage Carrier

AS,RMS=RMS voltage Sideband

PC=AC,RMS22-------(1)

PS=μ2AC,RMS24------(2)

(2)/(1),

10log10PC×103=x1

PC= 10(x1-30)10----------(3)

PS= 10(x2-30)10+10(x3-30)10-------(4)

(4)/(3),

PSPC=10(x2-30)10+10(x3-30)1010(x1-30)10-------(B)

(A)=(B),

μ22=10(x2-30)10+10(x3-30)1010(x1-30)10

μ=210(x2-30)10+10(x3-30)1010(x1-30)10

a). Sinusoidal signal

- When μ= 0.25,

From figure 04,

μ=210-52.9-3010+10-52.9-301010-34.9 -3010

μ=0.25

- When μ= 0.5,

From figure 05,

μ=210-46.8-3010+10-46.8-301010-34.9-3010

μ=0.51

- When μ= 1.0,

From figure 06,

μ=210-40.8-3010+10-40.8-301010-34.9-3010

μ=1.00

b). Square Wave signal

- When μ= 0.25,

From figure 11,

μ=210-60.5-3010+10-60.5-301010-42.9-3010

μ=0.26

- When μ= 0.5,

From figure 13,

μ=210-44.5-3010+10-44.5-301010-34.6-3010

μ=0.64

- When μ= 1.0,

From figure 12,

μ=210-39-3010+10-39-301010-19.6-3010

μ=1.21

c). Triangular pulse signal

- When μ= 0.25,

From figure 19,

μ=210-53.5-3010+10-54.5-301010-34.6-3010

μ=0.20

- When μ= 0.5,

From figure 21,

μ=210-48.5-3010+10-48.5-301010-14-3010

μ=0.40

- When μ= 1.0,

From figure 20,

μ=210-42.5-3010+10-42.5-301010-14-3010

μ=0.81

Efficiency of the modulated signal

Efficiency=Power of the sidebandsTotal power

For a sinusoidal wave,

μ=0.25,

η=10-52.9-3010+10-52.9-301010-52.9-3010+10-52.9-3010+ 10-34.9-3010

η = 3.07%

μ=0.5,

η=10-46.810+10-46.81010-46.810+10-46.810+10-34.910

η = 11.44%

μ=1.0,

η=10-40.810+10-40.81010-40.810+10-40.810+10-34.910

η =33.95%

Part 3: Simulation of AM Demodulation using a Synchronous Detector

# Lab 2: AM Synchronous Demodulation Simulation

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import butter, filtfilt

# Parameters

fm = 1000

fc = 20000

fs = 400000

duration = 0.005

t = np.linspace(0, duration, int(fs*duration), endpoint=False)

# 1. Generate Basic Signals (Part 1 of Procedure)

Am1 = 1.0

Ac = 2.0

m_t_basic = Am1 * np.cos(2 * np.pi * fm * t)

c_t = Ac * np.cos(2 * np.pi * fc * t)

# 2. Generate AM Signal (Part 2 of Procedure)

mu = 0.8

Am_mod = mu * Ac

m_t_mod = Am_mod * np.cos(2 * np.pi * fm * t)

# AM Signal

s_t = (Ac + m_t_mod) * np.cos(2 * np.pi * fc * t)

# 3. Synchronous Demodulation

local_carrier = np.cos(2 * np.pi * fc * t)

v_t = s_t * local_carrier

# Step B: Low Pass Filter (Specifics: 5kHz cutoff, 2nd order)

def butter_lowpass_filter(data, cutoff, fs, order=5):

nyq = 0.5 * fs

normal_cutoff = cutoff / nyq

b, a = butter(order, normal_cutoff, btype='low', analog=False)

y = filtfilt(b, a, data)

return y

# Cutoff = 5 kHz, Order = 2

demodulated_raw = butter_lowpass_filter(v_t, 5000, fs, order=2)

# Remove DC Component (0.5 * Ac) to recover AC message

demodulated_ac = demodulated_raw - np.mean(demodulated_raw)

# Scale to match original amplitude (Factor of 0.5 introduced by mixing)

demodulated_scaled = demodulated_ac * 2

# Plotting

plt.figure(figsize=(12, 14))

# 1. Message Signal

plt.subplot(5, 1, 1)

plt.plot(t * 1000, m_t_basic)

plt.title(f'Message Signal $m(t)$')

plt.ylabel('Amplitude (V)')

plt.grid(True)

# 2. Carrier Signal

plt.subplot(5, 1, 2)

plt.plot(t * 1000, c_t)

plt.title(f'Carrier Signal $c(t)$')

plt.ylabel('Amplitude (V)')

plt.grid(True)

# 3. AM Signal

plt.subplot(5, 1, 3)

plt.plot(t * 1000, s_t)

plt.plot(t * 1000, Ac + m_t_mod, 'g', linewidth=1, label='Upper Envelope')

plt.title(f'AM Signal $s(t)$ ($\mu=0.8$)')

plt.ylabel('Amplitude (V)')

plt.legend(loc='upper right')

plt.grid(True)

# 4. Product Signal v(t)

plt.subplot(5, 1, 4)

plt.plot(t * 1000, v_t)

plt.title(f'Product Signal $v(t) = s(t) \cdot c_{{LO}}(t)$')

plt.ylabel('Amplitude (V)')

plt.grid(True)

# 5. Demodulated Signal comparison

plt.subplot(5, 1, 5)

plt.plot(t * 1000, m_t_mod, 'r--',label='Original Message (Scaled)', linewidth=2, alpha=0.5)

plt.plot(t * 1000, demodulated_scaled, label='Demodulated Output')

plt.title('Demodulation Result (2nd Order LPF, $f_c=5$kHz)')

plt.xlabel('Time (ms)')

plt.ylabel('Amplitude (V)')

plt.legend(loc='upper right')

plt.grid(True)

plt.tight_layout()

Figure 25: Time domain representation of message signal, carrier signal, amplitude modulated signal, demodulated output signal and filter output signal