Part 3: Simulation of AM Demodulation using a Synchronous Detector  simulation session

OBSERVATIONS

Figure 29

Code

import numpy as np

import matplotlib.pyplot as plt

from scipy import signal

def simulate_synchronous_am_demodulation(

*,

A_m: float = 1.0,

f_m: float = 1_000.0,

A_c: float = 2.0,

f_c: float = 100_000.0,

mu: float = 0.8,

fs: float = 1_000_000.0,

duration: float = 0.005,

lpf_cutoff: float = 5_000.0,

butter_order: int = 2,

):

if fs <= 2 * max(f_c * 2, lpf_cutoff):

raise ValueError("Sampling frequency fs is too low for the chosen f_c/cutoff.")

if mu <= 0:

raise ValueError("Modulation index mu must be > 0.")

if lpf_cutoff >= fs / 2:

raise ValueError("LPF cutoff must be below Nyquist (fs/2).")

t = np.arange(0.0, duration, 1.0 / fs)

# 1) Message and carrier

m = A_m * np.cos(2 * np.pi * f_m * t)

c = A_c * np.cos(2 * np.pi * f_c * t)

# 2) AM signal: s(t) = A_c(1 + μ m_norm(t))cos(2π f_c t)

m_norm = m / A_m

s = A_c * (1.0 + mu * m_norm) * np.cos(2 * np.pi * f_c * t)

# 3) Synchronous demodulation

lo = np.cos(2 * np.pi * f_c * t)  # same frequency + phase as carrier

v = s * lo

# Low-pass filter (2nd order Butterworth, cutoff 5 kHz)

b, a = signal.butter(butter_order, lpf_cutoff / (fs / 2), btype="low")

v_lpf = signal.filtfilt(b, a, v)

# Recover message:

# v_lpf ≈ 0.5*A_c*(1 + μ m_norm(t))  -> remove DC and scale

m_hat = A_m * (2.0 / (A_c * mu)) * (v_lpf - 0.5 * A_c)

return {

"t": t,

"m": m,

"c": c,

"s": s,

"lo": lo,

"v": v,

"v_lpf": v_lpf,

"m_hat": m_hat,

"params": {

"A_m": A_m,

"f_m": f_m,

"A_c": A_c,

"f_c": f_c,

"mu": mu,

"fs": fs,

"duration": duration,

"lpf_cutoff": lpf_cutoff,

"butter_order": butter_order,

},

}

def plot_results(results: dict) -> None:

plt.style.use("seaborn-v0_8-darkgrid")

plt.rcParams["figure.figsize"] = (12, 10)

t = results["t"]

m = results["m"]

c = results["c"]

s = results["s"]

v = results["v"]

m_hat = results["m_hat"]

params = results["params"]

fig, axes = plt.subplots(5, 1, sharex=True)

axes[0].plot(t * 1e3, m, linewidth=2)

axes[0].set_title(r"Message signal $m(t)=A_m\cos(2\pi f_m t)$")

axes[0].set_ylabel("V")

axes[1].plot(t * 1e3, c, linewidth=1)

axes[1].set_title(r"Carrier signal $c(t)=A_c\cos(2\pi f_c t)$")

axes[1].set_ylabel("V")

axes[2].plot(t * 1e3, s, linewidth=1)

axes[2].set_title(r"AM signal $s(t)=A_c(1+\mu m_{norm}(t))\cos(2\pi f_c t)$")

axes[2].set_ylabel("V")

axes[3].plot(t * 1e3, v, linewidth=1)

axes[3].set_title(r"Mixer output $v(t)=s(t)\,c_{LO}(t)$")

axes[3].set_ylabel("V")

axes[4].plot(t * 1e3, m, label="Original m(t)", linewidth=2)

axes[4].plot(t * 1e3, m_hat, label=r"Demodulated $\hat{m}(t)$", linewidth=2, alpha=0.85)

axes[4].set_title(r"Recovered message after 2nd-order Butterworth LPF")

axes[4].set_xlabel("Time (ms)")

axes[4].set_ylabel("V")

axes[4].legend(loc="upper right")

fig.suptitle(

"Synchronous AM Demodulation (Coherent Detector)\n"

+ f"A_m={params['A_m']}V, f_m={params['f_m']/1e3:.1f}kHz, "

+ f"A_c={params['A_c']}V, f_c={params['f_c']/1e3:.1f}kHz, "

+ f"μ={params['mu']}, LPF={params['lpf_cutoff']/1e3:.1f}kHz",

y=0.995,

fontsize=12,

)

plt.tight_layout()

plt.show()

def main() -> None:

results = simulate_synchronous_am_demodulation()

plot_results(results)

if __name__ == "__main__":

main()